# SqueezeNet Standalone Accelerator SoC

## 1. Global AXI4 Memory Map

The system uses a 32-bit AXI4 crossbar with 3 Masters (SysCtrl, DVP Camera, Sys DMA) and 6 Slaves. The uppermost bits [31:29] route transactions to the respective Slaves.

- 0x0000_0000 - 0x1FFF_FFFF: DVP Camera Subsystem (S0)

  - 0x0000_0000: DVP-DMA Registers (atfox272/dma, CHN0)

  - 0x0001_0000: DVP-RX Controller Registers

- 0x2000_0000 - 0x3FFF_FFFF: QSPI Controller (S1)

  - 0x2000_0000 - 0x27FF_FFFF: QSPI Flash Memory (XIP Region)

  - 0x2800_0000 - 0x2FFF_FFFF: QSPI Configuration Registers

- 0x4000_0000 - 0x5FFF_FFFF: System DMA (S2) (atfox272/dma, CHN0 & CHN1)

  - 0x4000_0000 - 0x4000_003C: Sys-DMA Channel 0 Registers (Weight loading)

  - 0x4000_0040 - 0x4000_007C: Sys-DMA Channel 1 Registers (In-place Concat/Reshape)

- 0x6000_0000 - 0x7FFF_FFFF: Ping-Pong SRAM (S3)

  - 0x6000_0000 - 0x6001_3FFF: Bank A (80 KB)

  - 0x6001_4000 - 0x6002_7FFF: Bank B (80 KB)

- 0x8000_0000 - 0x9FFF_FFFF: Weight SRAM (S4) (8 KB)

- 0xA000_0000 - 0xBFFF_FFFF: SPI Slave / MCU Interface (S5)

- 0xC000_0000 - 0xDFFF_FFFF: Layer Descriptor SRAM (S6/Internal)

  - Note: In the current FSM, the descriptor SRAM is built into the sys_controller, but logically mapped here for MCU SPI programming.

## 2. SqueezeNet Descriptor Programming Framework

To execute a sequence of layers autonomously without a CPU, the external MCU writes an array of configuration structs into the Layer SRAM via the SPI interface before asserting the start signal. Each layer requires exactly 64 Bytes (16 words).

### Descriptor Struct Memory Map (64 Bytes per Layer)

- Word 00 (+0x00): {1'b_flip_bank, 7'b_n_passes, 8'b_cfg_stride, 16'b_cfg_kpat}

- Word 01 (+0x04): {8'b0, 24'b_cfg_base}

- Word 02 (+0x08): {8'b0, 24'b_cfg_chstep}

- Word 03 (+0x0C): {8'b0, 24'b_cfg_ystep}

- Word 04 (+0x10): {8'b0, 24'b_cfg_nch}

- Word 05 (+0x14): {8'b0, 24'b_cfg_nky}

- Word 06 (+0x18): {8'b0, 24'b_cfg_last}

- Word 07 (+0x1C): {8'b0, 24'b_cfg_x_tile_step}

- Word 08 (+0x20): {8'b0, 24'b_cfg_y_tile_step}

- Word 09 (+0x24): {8'b0, 24'b_cfg_ntx}

- Word 10 (+0x28): {8'b0, 24'b_cfg_nty}

- Word 11 (+0x2C): reqt_total_tiles

- Word 12 (+0x30): {15'b0, 1'b_maxpool_en, 16'b_pool_x_chunks}

- Word 13 (+0x34): {12'b0, 4'b_pool_stride, 16'b_pool_y_rows}

- Word 14 (+0x38): flash_src_addr

- Word 15 (+0x3C): {16'b0, 16'b_weight_words}

### Concat Descriptor Extensions

For SqueezeNet "Fire Modules" where Expand 1x1 and Expand 3x3 operate on the same Squeeze layer output, the DMA is used to manage in-place concatenation. The struct includes additional logical markers:

- sram_needs_concat: 1-bit flag indicating DMA Channel 1 should trigger.

- sram_concat_src, sram_concat_dst, sram_concat_len: DMA parameters for memory-to-memory moving inside the Ping-Pong SRAM.

- Zero-Copy Logic:

  - Squeeze 1x1: reads Bank A, writes Bank B (flip_bank_after = 0).

  - Expand 1x1: reads Bank B, writes Bank A channels 0-15 (flip_bank_after = 0).

  - Expand 3x3: reads Bank B, writes Bank A channels 16-31 (flip_bank_after = 1).

### MCU SPI Programming Sequence

The MCU acts as the SPI master. To program the SoC:

- Write Layer Descriptors: The MCU issues SPI writes to base address 0xC000_0000, writing 16 words per layer.

  - SPI Write Command: [0x02 Write CMD] + [32-bit Address] + [32-bit Data]

- Trigger Engine: The MCU writes the total number of layers to the sys_controller execution register.

  - mcu_spi_write32(0x1000_0000, 0x0000_0003)

- Interrupt Handling: The SoC runs autonomously (fetching chunks from the DVP camera and weights from QSPI). Upon completing the final layer, it pulses host_irq, alerting the MCU to read the final classification ID.