Edge-CNN Accelerator SoC

How the system works - from the basics to every file

Project: /home/ms/vlsi/sys_v1    |    State: October 2026 (design version v6)

Camera in  ->  SqueezeNet-1.1 inference on chip  ->  class number out on pins / SPI

# Contents

- What this chip does (the one-page picture)

- Background: the ideas you need (CNNs, SqueezeNet, int8 quantisation, AXI)

- The SoC at a glance: blocks, pins and the memory map

- One inference, step by step

- The blocks in detail

- Row-streaming at 224 x 224 (Option A)

- Programming model: CSR registers and layer/job descriptors

- Software side: data generators and the fine-tuning pipeline

- How the design is verified (testbenches and results)

- File-by-file reference

- Current status, limitations and next steps

- Glossary

# 1. What this chip does

This system-on-chip (SoC) takes pictures from a small camera (an OV7670-type module), runs a neural network that recognises what is in the picture, and reports the answer as a class number. It needs no processor of its own: a hardware state machine runs the whole network. A small external microcontroller (MCU) only configures it once and starts it, over a 4-wire SPI link.

The network is SqueezeNet-1.1, a small image-classification network. Its weights (about 0.7-1.2 MB as 8-bit numbers) live in an external QSPI flash chip and are streamed into the chip while it works. The pictures and all intermediate results stay inside the chip, in 160 KB of on-chip SRAM.

In plain words, the flow is:

- The MCU writes a short program (the list of layers / jobs) and some settings over SPI, then writes START.

- The chip turns on the camera. Pixels arrive row by row; the chip keeps only the centre 224 x 224 square, converts the colours to 8-bit numbers and writes them into on-chip memory.

- As soon as enough rows have arrived, the compute engine starts working on them (it does not wait for the whole picture).

- For every layer the chip copies that layer's weights from the flash, multiplies them with the image data on a 4 x 4 array of multipliers, rescales the results back to 8 bits, applies ReLU and max-pooling, and writes the output back to on-chip memory for the next layer.

- After the last layer a small 'head' picks the class with the highest score. The class number appears on 7 output pins (with a 'valid' pin) and in a register the MCU can read; an interrupt pin tells the MCU the answer is ready.

Status: everything in this document is implemented in RTL and runs in Verilator simulation. Verified so far: the whole SoC path with conv1 + max-pool + the complete first Fire module (on an image patch), and full-resolution 224 x 224 row-streaming of conv1 + pool + fire2-squeeze directly from a simulated 30 fps camera. Running the entire network end-to-end is the next step (section 11).

# 2. Background: the ideas you need

## 2.1 What a convolutional neural network (CNN) computes

A picture is stored as numbers: for every pixel there is a red, a green and a blue value. A CNN transforms this 3-channel picture through a chain of layers into a list of scores, one per class ('dog', 'cup', ...). The highest score is the answer.

Convolution: a small window of weights (for example 3 x 3 pixels x all input channels) slides over the picture. At every position the pixel values under the window are multiplied by the weights and added up - a multiply-accumulate (MAC). Each output channel has its own set of weights, so a layer with 64 output channels produces 64 new 'pictures' (feature maps).

Stride: how far the window moves each step. Stride 2 halves the width and height.

Padding: imaginary pixels around the border so the window can also be centred on edge pixels.

ReLU: negative results are replaced by zero.

Max-pooling: keeps only the largest value in each small window (here 3 x 3, stride 2); it shrinks the picture and keeps the strongest responses.

1 x 1 convolution: a window of a single pixel - it mixes channels without looking at neighbours.

Global average pooling + argmax: the last layer's maps are averaged per class and the largest is chosen.

## 2.2 SqueezeNet-1.1 and the Fire module

SqueezeNet is built to be small. After a first 3 x 3 convolution (conv1, 3 -> 64 channels, stride 2) and a max-pool, it uses eight 'Fire modules'. A Fire module has:

Squeeze: a 1 x 1 convolution that reduces the channel count (e.g. 64 -> 16).

Expand: two parallel convolutions on the squeezed data - a 1 x 1 (16 -> 64) and a 3 x 3 with padding 1 (16 -> 64).

Concat: the two expand outputs are stacked into one map of 128 channels.

Three max-pools shrink the 224 x 224 input to 111, 55, 27 and 13 pixels. The final 1 x 1 convolution (conv10) produces one map per class.

## 2.3 Why 8-bit integers (quantisation)

Real networks are trained with floating-point numbers. Hardware is far smaller and faster with 8-bit integers. Each tensor is therefore stored as real_value = scale x (q - zero_point), where q is an 8-bit integer. The model file artifacts/squeezenet1_1.onnx is already quantised this way (weights: signed int8 per output channel; activations: unsigned uint8).

Inside the chip every activation is stored as signed int8 = uint8 - 128 (just flipping the top bit). This lets one signed multiplier handle everything; the 'minus 128' is folded into the bias.

Requantisation: the MAC result is a 24-bit integer. To get back to 8 bits the chip computes (acc + bias) x M >> shift + zero_point and clips it. M (a 31-bit fixed-point fraction) and shift replace the floating-point ratio scale_in x scale_w / scale_out. This is the same arithmetic as TensorFlow-Lite, so results match a software reference bit-for-bit.

ReLU for free: with uint8 outputs whose zero point is 0, ReLU is simply 'clip at the zero point'.

## 2.4 AXI, SPI, DVP, QSPI in one minute

AXI4: the on-chip bus standard used between blocks. A 'master' starts reads/writes, a 'slave' answers. An interconnect routes each transfer by address. Bursts move several 32-bit words per request.

SPI: a simple 4-wire serial link (clock, chip-select, data in, data out) used by the MCU.

DVP: the parallel camera interface: pixel clock, 8 data bits, line-valid (HREF) and frame-sync (VSYNC). An RGB565 pixel arrives as two bytes.

QSPI flash: serial flash memory read 4 bits per clock. 'XIP' (execute in place) mode lets the chip read it like ordinary memory at an address.

# 3. The SoC at a glance

## 3.1 Block diagram (who talks to whom)

OV7670 camera ──DVP──► dvp_rx_core ──pixels──► cam_frontend ─┐ (M1)

│

MCU ──SPI──► spi_slave_axi ───────────────────────────────────┤ (M3)

│

sys_controller (the 'CPU-less' sequencer) ────────────────────┤ (M0)

│

sys-DMA (weights flash -> weight SRAM) ───────────────────────┤ (M2)

▼

AXI4 interconnect  (4 masters x 6 slaves)

┌───────────┬────────────┬──────────┬────────────┬──────────┬──────────┐

S0 DVP regs S1 QSPI      S2 DMA regs S3 act_mem  S4 weight  S5 sys_csr

+ flash XIP              160 KB      SRAM 32 KB  registers

│  ▲          │

read port  ▼  │ write    ▼

┌──────── compute_top ──────────┐

│ mme (feeder + 4x4 PE array)    │

│ requant_and_pack -> maxpool    │

│ -> out_writer                  │

└────────────────────────────────┘

result_o[6:0], result_valid_o, busy_o, host_irq  ◄── sys_csr / head

## 3.2 Top-level pins (accelerator_top.sv)

| Pin(s) | Dir | Meaning |
| --- | --- | --- |
| clk | in | system clock (100 MHz in simulation) |
| rst_n | in | active-low reset; asserted at once, released synchronously (2-flop synchroniser) |
| dvp_pclk_i, dvp_vsync_i, dvp_hsync_i, dvp_href_i, dvp_d_i[7:0] | in | camera pixel bus |
| dvp_xclk_o, dvp_pwdn_o | out | camera master clock and power-down |
| qspi_sck, qspi_cs_n, qspi_io[3:0] | out/inout | weight flash (quad SPI, XIP reads) |
| mcu_spi_sck, mcu_spi_cs_n, mcu_spi_mosi, mcu_spi_miso | in/out | MCU control and read-back |
| host_irq | out | 'inference done', stays high until the MCU clears it |
| result_o[6:0] | out | class number in binary (0-127); enable with PIN_CFG |
| result_valid_o | out | result_o is valid (cleared by the next START) |
| busy_o | out | an inference is running |

## 3.3 Memory map (addresses as seen by AXI masters and by the MCU over SPI)

| Address | Slave | What is there |
| --- | --- | --- |
| 0x0000_0000 | S0 | DVP DMA registers (unused, kept for the register map) |
| 0x0001_0000 | S0 | DVP-RX controller registers (camera enable, size, start) |
| 0x2000_0000 .. | S1 | QSPI flash, memory-mapped (XIP) - weights live here |
| 0x2800_0000 .. | S1 | QSPI controller registers |
| 0x4000_0000 | S2 | sys-DMA registers (one address per register) |
| 0x6000_0000 - 0x6002_7FFF | S3 | act_mem: 160 KB activation memory (old bank A = first 80 KB, bank B = second) |
| 0x8000_0000 | S4 | weight SRAM, 32 KB (8192 words) |
| 0xA000_0000 | S5 | system control / status registers (section 7.1) |
| 0xA001_0000 | S5 | layer / job descriptors: job x 128 + word x 4 (section 7.2) |

The interconnect decodes address bits [31:29] to pick the slave. Each master's ID is extended by 2 bits so responses find their way back; every slave therefore echoes the request ID.

# 4. One inference, step by step

- MCU programs the chip over SPI: the job descriptors (0xA001_0000..), camera settings (crop window, stripe slots), classifier-head settings, then writes the number of jobs to CTRL (0xA000_0000) = START.

- Flash set-up: sys_controller writes the QSPI registers so the flash can be read as memory (quad read 0x6B, XIP window at 0x2000_0000).

- Camera set-up: it programs the DVP-RX core (frame size from the CSR, RGB565 mode) and arms cam_frontend, then starts capture.

- Pixels arrive: cam_frontend keeps only the crop window, converts each RGB565 pixel to three int8 values and writes every finished row into a 'stripe slot' in act_mem. It counts finished rows.

- Job loop: for every job the controller (a) copies a chunk of the layer's weights from flash into the weight SRAM with the sys-DMA, (b) latches the layer geometry from the descriptor, (c) if the job needs camera rows, waits until enough rows are written, (d) optionally fills the borders of a padded output map, (e) for each group of 4 output channels ('pass') reads the 12 requantisation numbers from the weight SRAM and pulses START of the compute engine, waits until it is done, (f) loads the next weight chunk when the current one is used up, (g) tells the camera front end a stripe slot is free again.

- Compute engine (per pass): the feeder fetches the input windows from act_mem, the 4 x 4 PE array multiplies them with 4 channels of weights, requant_and_pack turns the 24-bit sums into int8 with ReLU, maxpool_inline pools them (if enabled) and out_writer stores the words at the right place in act_mem.

- Head: after the last job, the controller sums every class map and keeps the largest (argmax).

- Done: the class number goes to RESULT and to the result pins, host_irq rises, the chip goes idle. The next START captures a new frame.

# 5. The blocks in detail

## 5.1 accelerator_top - the chip

Instantiates and wires every block: the interconnect (4 masters, 6 slaves), the DVP subsystem and camera front end, the sys-DMA, the QSPI controller with its tri-state flash pads, act_mem, the weight SRAM, the compute engine, the MCU SPI bridge, the CSR block and the controller. Small glue in the top:

reset synchroniser (asynchronous assert, synchronous release);

word-to-byte address adapter for the sys-DMA (the DMA IP counts in 32-bit words, the bus in bytes);

AXI-ID echo for slaves that have no ID ports (QSPI) or too-narrow IDs (DMA registers), so responses are routed back to the right master;

WSTRB = all ones (every master writes whole 32-bit words);

result pins driven from the CSR block when PIN_CFG[0] = 1.

## 5.2 MCU SPI port - spi_slave_axi (bus master M3)

The MCU's window into the chip. It turns SPI frames into AXI reads and writes, so the MCU can reach every address in the memory map (registers, descriptors, even the SRAMs - useful for reading out detection results).

| Frame (CS low for the whole frame) | Effect |
| --- | --- |
| 0x02 ¦ address[31:0] ¦ data[31:0] { ¦ data ... } | write; the address auto-increments by 4 for extra words |
| 0x03 ¦ address[31:0] ¦ 8 dummy clocks ¦ data[31:0] on MISO | read one word |

Mode: SPI mode 0 (sample on rising edge), MSB first.

Speed: SCK must be at most clk / 8 (SCK, CS and MOSI are over-sampled with synchronisers).

Read timing: the AXI read is issued when the address is complete and must return within the 8 dummy clocks; a late read returns 0xDEADBEEF and is counted (rd_late).

Status: built and verified in the system tests (ID read, all descriptor/CSR writes, result / status / SRAM read-back, IRQ clear). Not yet exercised: multi-word write bursts. Known limits: one word per read; MISO is always driven (no tri-state yet, so it cannot share a bus with other SPI slaves).

## 5.3 System registers - sys_csr (slave S5)

A small register file: start/status/IRQ, the result, the classifier-head and camera settings, and the descriptor window that is forwarded to the controller. Full table in section 7.1. host_irq = (inference done) AND IRQ_EN; writing 1 to IRQ clears it.

## 5.4 The sequencer - sys_controller (bus master M0)

This replaces a CPU. It is one large state machine plus two tiny AXI engines (single-word write and read). It holds up to 64 job descriptors (32 words each) and walks through them. Main state groups:

| States | What happens |
| --- | --- |
| ST_RESET, ST_WAIT_MCU_PROGRAM | idle until the MCU writes a non-zero job count to CTRL |
| ST_QSPI_* | set clock divider, quad mode, dummy cycles, XIP window, read command, enable XIP |
| ST_DVP_DRC_* | program the DVP-RX core (enable, power, stream mode, IRQ mask, width, height, format, start); arm cam_frontend |
| ST_WAIT_CHUNK | start the job list (the camera keeps running in parallel) |
| ST_LAYER_AXI_GATE, ST_DMA_WLOAD_* | program the sys-DMA: flash address of this weight chunk -> weight SRAM, length; wait for its done-interrupt |
| ST_LAYER_COMPUTE_GATE | copy the job's geometry from the descriptor to the compute engine |
| ST_WAIT_CAM | row-streaming: wait until the camera has written the rows this job needs |
| ST_FILL, ST_FILL_WAIT | fill the borders of a padded output map with the zero point |
| ST_PASS_SET_PARAMS | read bias[4], M[4], shift[4] of this pass from the weight SRAM |
| ST_PASS_START, _WAIT_DONE, _FLUSH, _NEXT | pulse compute start, wait for the last tile, let the pooling pipeline drain, next pass or next weight chunk |
| ST_LAYER_BANKFLIP, ST_LAYER_NEXT | job end: release a camera slot if asked; next job |
| ST_HEAD_RD | classifier head: sum each class map, keep the maximum |
| ST_DONE | publish result, pulse 'done', go idle |

sys_axi_ctrl selects who owns the weight SRAM: the AXI side (DMA loads, parameter reads) or the compute engine (during a pass).

## 5.5 Camera path - DVP-RX core and cam_frontend (bus master M1)

dvp_rx_core (third-party IP in rtl/dvp-rx) samples the DVP bus with the pixel clock, moves pixels into the system clock domain and outputs a stream of 32-bit words (two RGB565 pixels each). Its own DMA is no longer used.

cam_frontend (new) does everything between the camera and the memory:

Crop: counts x and y and keeps only the crop window (default 224 x 224 at x=48, y=8 of 320 x 240) - extra pixels are never written to SRAM.

Colour conversion: RGB565 -> 8-bit R, G, B by bit replication, then XOR 0x80 to get signed int8.

Row-planar layout: a finished row is written as R[224] G[224] B[224] (672 bytes). The conv1 feeder reads this directly with channel step 224 and row step 672 - no re-ordering of the picture is needed.

Stripe slots: rows are grouped into stripes (16 rows + 3 'halo' rows shared with the next stripe) stored in a ring of slots (3 slots). Halo rows are written to both slots, so each stripe is contiguous for the engine.

Handshake: rows_done counts completed rows (all writes acknowledged); the controller releases a slot with 'consume'. If the camera would overwrite a slot still in use, the row is dropped and 'overrun' is flagged (the camera is too fast for the engine).

RAW mode: bytes are stored exactly as received (used by the patch test).

Implementation: two line buffers (fill one while writing the other), each made of three byte-enable word planes - suitable for block RAM. AXI bursts of up to 16 words, never crossing a 64-byte line.

## 5.6 Weights path - QSPI controller, flash and sys-DMA (master M2)

Flash: W25Q16JV (2 MB). In simulation the vendor model rtl/flash_model is loaded with flash.hex.

QSPI controller (rtl/axi_qspi_controller, third-party): after the XIP set-up, any AXI read in 0x2000_0000.. becomes a quad read (command 0x6B) from the flash; bursts are supported.

sys-DMA (rtl/dma, atfox272/dma IP): copies a block of words from flash (XIP) to the weight SRAM. Its registers use one address per register and its addresses are word addresses (the top level converts them).

Weight blob format (per layer, in flash): one block per pass of 4 output channels = k_len weight words (word t = the 4 channels' weights for step t, one byte each) + 4 bias + 4 multiplier + 4 shift words. A descriptor field says how many blocks to load per DMA, so the 32 KB weight SRAM holds as much of a layer as fits.

## 5.7 Weight SRAM - wbuf_axi_wrapper (slave S4)

32 KB (8192 x 32-bit). Written by the DMA through axi4_sram_slave; read by the compute engine (one word per cycle) or by the controller (parameters). A multiplexer controlled by sys_axi_ctrl picks the owner.

## 5.8 Activation memory - act_mem (slave S3)

All pictures and feature maps live here: one 160 KB address space built from five 32 KB sub-banks, each with one read and one write port (the standard FPGA block-RAM / 2-port SRAM shape).

Compute read (the feeder): fixed 1-cycle latency, never stalled.

Compute write (out_writer): never stalled. If it collides with an AXI write to the same sub-bank it waits in a 16-entry in-order queue and is written on the next free cycle.

AXI (camera, DMA, SPI, controller): bursts; writes are never back-pressured (the interconnect mis-handles that, see section 11); reads wait only while compute reads the same sub-bank.

Result: the camera can fill stripe slots while the engine computes on other memory.

## 5.9 Compute engine - compute_top

### mme - the matrix-multiply engine

ifmap_feeder: an on-the-fly 'im2col'. For each output tile of 4 pixels it reads the needed input words (channels x kernel rows x columns, with stride, dilation and an arbitrary start offset) and builds four lanes of values, one per output pixel. Five counters walk the whole layer (x tiles, y tiles, channels, kernel rows, words) from one START pulse.

fifo_controller + lane_fifo: small FIFOs for the activation lanes and the weight columns; staggers them diagonally so the systolic array gets data at the right cycle; freezes the array if a FIFO runs dry.

pe4x4 / pe_v2: 16 processing elements (4 pixels x 4 output channels). Each does one int8 x int8 multiply per cycle (Booth_Multiplier: Booth_Encoder + Wallace_Tree of CSA/FA + KSA adder) and accumulates into 24 bits with saturation; after K steps it outputs the sum. 16 MACs per clock.

### requant_and_pack + tflite_requantizer

Four requantisers turn the 24-bit sums of one tile into int8 ((acc+bias) x M >> shift + z, ReLU clip) and pack the 4 pixels of one channel into one 32-bit word.

### maxpool_inline

Pools the stream on the fly with 3 x 3 / stride 2 windows using row delay lines. Two modes: pad 1 (windows centred on even pixels, ShuffleNet style) and pad 0 (SqueezeNet: window 2i..2i+2). It also handles tile padding at the right border.

### out_writer

Decides where each output word goes: base + (channel offset + channel) x channel stride + (row + pad) x row stride + pad + x. This one rule gives:

padded output maps (a one-word border left/right, one row top/bottom) for 3 x 3 / pad-1 layers - the border is filled with the zero point by a built-in fill engine;

Fire concat without copying: expand-3x3 simply writes after expand-1x1's channels (channel offset);

widths that are not a multiple of 4 (55, 27, 13): pixels past the valid width are written as the zero point, which acts as 'minus infinity' for pooling and zero for the next layer.

## 5.10 Classifier head and result pins

After the last job the controller reads each class map (head_words words at HEAD_BASE + class x head_words x 4) over AXI, adds up all its int8 values and keeps the largest (the lowest class number wins a tie). Because every class map has the same size, this equals global average pooling + argmax. The winner goes to RESULT/SCORE, result_o[6:0] and result_valid_o; host_irq rises.

## 5.11 Interconnect and common library

rtl/interconnect is a third-party AXI4 crossbar: dispatchers per master (dsp_*), arbiters per slave (sa_*), an outstanding-transaction limit and a 4 KB-boundary splitter. rtl/common holds the small building blocks used by the IP (FIFOs, skid buffers, arbiters, encoders, edge detectors).

# 6. Row-streaming at 224 x 224 (Option A)

A full 224 x 224 x 3 picture is 147 KB and the biggest SqueezeNet feature maps are up to 387 KB - far more than the 160 KB on chip. Instead of storing whole maps, the chip processes horizontal stripes and keeps only what the next layer needs.

## 6.1 What runs today (stage 1)

| Item | Value |
| --- | --- |
| Camera | 320 x 240 RGB565 at 30 fps (pixel clock 5 MHz in the test) |
| Crop | 224 x 224 at (48, 8), row-planar int8, 672 bytes per row |
| Stripes | 14 stripes of 4 pooled rows (last one 3); conv1 computes 2n+1 rows per stripe (1 row overlap) |
| Camera slots | 3 slots x (16 + 3 halo) rows = 38 KB at act_mem word 0 |
| Jobs | 28 = 14 x { conv1+ReLU+max-pool (waits for its rows, frees its slot), fire2-squeeze } |
| Pool stripe buffer | 64 ch x 4 rows x 56 px at word 9600 |
| fire2-squeeze output | 16 x 55 x 55, stored padded (16 x 57 x 64 bytes) at word 13312 - ready for the 3 x 3 expand |
| Measured (simulation) | first stripe 3.6 ms after the frame starts; the stage ends 3.5 ms after the last row; no overrun; bit-exact vs the whole-image computation |

## 6.2 Plan for the rest of the network

Keep the small 'squeeze' maps whole (at most 58 KB) and stream the large expand outputs stripe by stripe straight into the next squeeze (or into max-pool + squeeze).

Move the job list into flash (the full network needs more than 64 jobs) and fetch jobs one at a time.

A compiler script turns the fine-tuned ONNX model into jobs, the memory plan and the flash image.

# 7. Programming model

## 7.1 System registers (0xA000_0000)

| Offset | Name | Access | Fields |
| --- | --- | --- | --- |
| 0x00 | CTRL | W/R | [7:0] number of jobs; writing it starts an inference |
| 0x04 | STATUS | R | [0] busy [1] done [2] result valid [8] DVP-RX trap [9] DVP-DMA trap [10] sys-DMA trap or act_mem queue overflow [11] camera overrun [23:16] job [31:24] pass |
| 0x08 | IRQ | R/W1C | [0] done (write 1 to clear) |
| 0x0C | IRQ_EN | RW | [0] enable host_irq (reset 1) |
| 0x10 | RESULT | R | [15:0] class [31] valid |
| 0x14 | SCORE | R | score of the winning class |
| 0x18 | HEAD_CFG | RW | [15:0] number of classes (0 = no head) [31:16] words per class map |
| 0x1C | HEAD_BASE | RW | AXI byte address of class 0's map |
| 0x20 | PIN_CFG | RW | [0] drive result_o / result_valid_o (reset 1) |
| 0x24 | ID | R | 0x5ACE0001 |
| 0x40 | CAM_FRAME | RW | {height, width} of the sensor frame (reset 240, 320) |
| 0x44 | CAM_CROP_XY | RW | {y0, x0} (reset 8, 48) |
| 0x48 | CAM_CROP_WH | RW | {h, w}, w multiple of 4 (reset 224, 224) |
| 0x4C | CAM_MODE | RW | [0] 1 = RAW bytes, 0 = RGB565 -> R/G/B row planes |
| 0x50 | CAM_BUF | RW | AXI byte address of slot 0 |
| 0x54 | CAM_SLOT | RW | bytes between slots |
| 0x58 | CAM_STRIPE | RW | {slots[31:24], halo rows[23:16], rows per stripe[15:0]} |
| 0x5C | CAM_ROWSTR | RW | bytes between rows |
| 0x60 | CAM_STATUS | R | [15:0] rows written [30] frame done [31] overrun |

## 7.2 Job descriptor (32 words per job at 0xA001_0000 + job x 128)

| Word | Field(s) |
| --- | --- |
| W00 | {flip (ignored), n_passes[6:0], stride[7:0], kernel pattern[15:0]} |
| W01 | input base - absolute act_mem element (byte) address |
| W02 / W03 | channel step / row step of the input (elements) |
| W04 / W05 / W06 | input channels / kernel rows / last element index of a tile row |
| W07 / W08 | x / y tile step (elements) |
| W09 / W10 | tiles in x / tiles in y |
| W11 | total tiles (informative) |
| W12 | [15:0] pool x chunks [16] max-pool enable [17] ReLU |
| W13 | [15:0] pool rows [19:16] pool stride [20] pool pad 0 (SqueezeNet) |
| W14 | flash byte offset of the layer's first weight block |
| W15 | [15:0] weight words (informative) [25:16] k_len (MACs per output, up to 1023) |
| W16 / W17 | output base / channel stride (words) |
| W18 | {data words per row, row stride} (words) |
| W19 | [15:0] channel offset (Fire concat) [16] padded output |
| W20 | {output rows, valid width} (valid width 0 = no masking) |
| W21 | {n_passes override[23:16], z_out[15:8], passes per weight load[7:0] (0 = all)} |
| W22 | number of output frames whose border is filled before the first pass |
| W23 | [0] wait for camera rows [1] release a camera slot at the end |
| W24 | camera rows needed |
| W25..W31 | reserved |

# 8. Software side

| Script / data | What it does |
| --- | --- |
| scripts/gen_sqz_fire2.py | Runs the original quantised SqueezeNet (ONNX) on artifacts/dog.jpg, extracts integer weights and requant parameters, writes the flash image, job descriptors, input patch and golden outputs for tb_system_sqz (conv1+pool+fire2 on a 65 x 65 patch). Contains the bit-exact Python model of the hardware arithmetic. |
| scripts/gen_cam_stage1.py | Builds a 320 x 240 RGB565 camera frame from dog.jpg, folds the input normalisation into conv1 and re-quantises it, plans 14 stripes / 3 slots, writes 28 jobs, the flash image and the full-image golden map for tb_system_cam. |
| ml/finetune_squeezenet.py | Run on Colab/Kaggle (GPU). Fine-tunes ImageNet SqueezeNet-1.1 on 20-30 classes with camera-faithful pre-processing (320 x 240, RGB565, centre crop 224), folds mean/std into conv1, exports int8 QDQ ONNX with the hardware's rules enforced, reports accuracy, exports raw test frames. Not run locally. |
| hex/sqz_fire2/*, hex/cam_stage1/* | Generated data read by the testbenches ($readmemh): frame, flash image, descriptors, expected memory contents. |
| artifacts/squeezenet1_1.onnx + .data | The quantised reference model (graph + external weights). |
| artifacts/dog.jpg | Test picture (classified as Samoyed, ImageNet class 258). |

# 9. How the design is verified

Simulator: Verilator 5.036 (make targets in Makefile). Every system test drives only the chip's pins (SPI, DVP, flash pins) - nothing is forced from inside except loading the flash model.

| make target | Testbench | What it proves | Result |
| --- | --- | --- | --- |
| make system_cam | tb/tb_system_cam.sv | Full-resolution row-streaming: 320 x 240 camera, on-chip crop, 14 stripes during capture, every row in its slot, no overrun, 16 x 55 x 55 squeeze map bit-exact, head on pins + SPI | PASS |
| make system_sqz | tb/tb_system_sqz.sv | Whole SoC path on a patch: SPI programming, RAW camera, chunked weight DMA from flash, conv1+pool+fire2 (padding, concat, width masking, K = 144), head, IRQ, two inferences back to back; ONNX within 1 LSB | PASS |
| make conv_default | tb/conv_tb.sv | Compute engine alone: depthwise, pointwise, dense 3 x 3, stride 2, dilation, K = 576, autonomous 5-counter mode, with and without random stalls | PASS (3620 tiles, 0 errors) |
| make unit | tb/tb_units.sv | lane FIFO and PE unit tests | FIFO pass; PE saturation test fails (expects a 32-bit accumulator, design uses 24-bit) - pre-existing |

Golden data: (1) a bit-exact Python model of the hardware - must match exactly; (2) ONNX Runtime on the reference model - differences of 1 LSB are expected because ONNX rescales in floating point.

# 10. File-by-file reference

Files compiled for the SoC are listed in filelist_system.f. 'New' = written for this SoC; 'IP' = third-party block used as-is (with the fixes noted).

## 10.1 Top level and system control (project RTL, rtl/)

| File | Kind | Purpose |
| --- | --- | --- |
| accelerator_top.sv | top | the chip: pins, interconnect, all blocks, reset synchroniser, address/ID glue |
| sys_controller.sv | new/updated | CPU-less sequencer: QSPI/camera set-up, job loop, weight DMA, passes, camera waits, head, done |
| sys_csr.sv | new | control/status/result/camera/head registers + descriptor window (slave S5), host_irq |
| spi_slave_axi.sv | new | MCU SPI slave -> AXI master (M3) |
| cam_frontend.sv | new | crop, RGB565 -> int8, row-planar stripes into act_mem (M1) |
| act_mem.sv | new | 160 KB unified activation memory, compute + AXI ports (S3) |
| wbuf_axi_wrapper.sv | updated | weight SRAM with AXI slave + compute read port (S4) |
| axi4_sram_slave.sv | updated | simple AXI4 slave front for an SRAM (used by the weight SRAM), ID echo |
| buffer.sv | existing | synchronous single-read SRAM model used for the weight SRAM |
| global.svh | existing | global parameters: 8-bit data, 4 x 4 array, 4 values per word, 24-bit accumulator, KLEN_W = 10 |

## 10.2 Compute engine (rtl/)

| File | Purpose |
| --- | --- |
| compute_top.sv | wraps mme, requant_and_pack, maxpool_inline and out_writer |
| mme.sv | matrix-multiply engine: feeder + FIFOs + 4 x 4 array, tile/layer done |
| ifmap_feeder.sv | on-the-fly im2col address generator and lane builder (5 counters) |
| fifo_controller.sv | lane FIFOs, weight fetch, diagonal stagger, freeze logic |
| fifo.sv (module lane_fifo) | small FIFO for one lane / column |
| pe4x4.v | 4 x 4 grid of PEs with result buses |
| pe_v2.sv | one processing element: 3-stage int8 MAC, 24-bit saturating accumulator |
| Booth_Multiplier.v, Booth_Encoder.v, Wallace_Tree.v, CSA.v, FA.v, KSA.v | 8 x 8 signed multiplier: radix-4 Booth encoding, Wallace tree of carry-save adders, Kogge-Stone final adder |
| requant_and_pack.sv | 4 requantisers + packing of 4 pixels per word |
| tflite_requantizer.sv | TFLite fixed-point requantisation and ReLU clip |
| maxpool_inline.sv | streaming 3 x 3 / stride-2 max-pool, pad 1 or pad 0 |
| out_writer.sv | output placement (strides, padding, concat offset, width mask) + border fill |

## 10.3 Camera receiver IP (rtl/dvp-rx/)

| File | Purpose |
| --- | --- |
| dvp_dma_wrapper.v | DVP-RX core + its register map and DMA; now exports the pixel stream to cam_frontend |
| dvp_rx_core.v | camera receiver top: pixel-clock sync, data FIFO, control, resizer, aligner, registers |
| drc_pclk_sync.v | brings the camera pixel clock events into the system clock domain |
| drc_dvp_data_fifo.v | buffers incoming bytes with sync/line flags |
| drc_ctrl_state.v, drc_cs_state_machine.v | capture state machines (frame/line tracking, start/stop, errors) |
| drc_resizer_mod.sv, drc_frm_downscaler_mod.sv, drc_pxl_grayscaler.v | optional grayscale and 2 x 2 downscale (bypassed here) |
| drc_mem_aligner.v | packs 16-bit pixels into 32-bit stream words |
| drc_regmap_mod.v | camera control registers (enable, size, mode, start) |
| drc_xclk_gen.v | generates the camera master clock |

## 10.4 Flash interface IP (rtl/axi_qspi_controller/, rtl/flash_model/)

| File | Purpose |
| --- | --- |
| axi_qspi_wrapper.sv | QSPI controller top: register interface + memory (XIP) interface + core |
| axi_qspi_axi4_interface.sv | configuration registers (mode, dummy, XIP base/mask/command ...) |
| axi_qspi_mem_interface.sv | turns AXI read bursts in the XIP window into flash reads |
| axi_qspi_core.sv | serial engine: command, address, dummy and data phases on 1/2/4 lines |
| axi_qspi_fifo.sv, axi_qspi_pkg.sv, axi_qspi_defines.svh | FIFO, constants (register offsets, commands) |
| sram_to_axi4_bridge.sv | used only by the reference testbench |
| flash_model/W25Q16JV_verilator_compat.v | vendor behavioural model of the 2 MB flash (simulation only) |

## 10.5 DMA IP (rtl/dma/, atfox272/dma)

| File(s) | Purpose |
| --- | --- |
| axi_dma.v | DMA top: register map, descriptor queue, channel manager, scheduler, data mover |
| adma_reg_map.v | registers (one address per register), submit FIFO, status |
| adma_desc_queue.v | descriptor queue - fixed: the done flag of every slot is now cleared on reuse (before, the 2nd use of slot 1 never raised its interrupt) |
| adma_chn_man.v, adma_cm_*.v | per-channel management: transfer split, fetch, done status, interrupts |
| adma_atx_sched.v, adma_as_*.v | splits transfers into AXI bursts and arbitrates channels |
| adma_data_mover.v, adma_dm_*.v | AXI read/write engines, stream ports, data buffer |
| axi_dma.sdc, axi_dma_filelist.f | timing constraints / original file list (not used by the build) |

## 10.6 Interconnect IP (rtl/interconnect/) and common library (rtl/common/)

| File(s) | Purpose |
| --- | --- |
| axi_interconnect.v | 4 x 6 AXI4 crossbar top |
| ai_dispatcher.v, dsp_*.v | per-master side: route requests to slaves, collect responses |
| ai_slave_arbitration.v, sa_*.v | per-slave side: arbitrate masters, order write data, return B/R by ID |
| splitting_4kb_masker.v | keeps bursts from crossing 4 KB boundaries |
| common/fifo/*, common/skid_buffer/* | FIFOs (sync, async, up/down-sizing) and skid buffers |
| common/arbiter/iwrr/* | weighted round-robin arbiter |
| common/encoder/*, decoder/*, converter/*, edgedet/*, splitter/*, memory/mem.v, reorder_buffer/* | small helpers (one-hot/priority encoders, Gray code, edge detect, ...) |
| common/adapter/axi/axi4_ctrl.v | generic AXI register-file adapter used by the DMA and DVP IP |

## 10.7 Testbenches, references and build files

| File | Purpose |
| --- | --- |
| tb/tb_system_cam.sv | full-resolution camera row-streaming system test |
| tb/tb_system_sqz.sv | conv1 + pool + fire2 system test, two inferences |
| tb/conv_tb.sv | compute-engine test with many conv shapes and random stalls |
| tb/tb_units.sv, tb/trace_tb.sv, tb/tb_ifmap_feeder.sv | unit tests / trace of the PE, FIFO, feeder |
| tb/tb_accelerator_*.sv, tb/tb_shufflenet_real.sv, tb/tb_system.sv | older tests for the previous compute-only top (ShuffleNet era) - do not build with the current top |
| reference/tb_dvp_dma.sv, tb_qspi_subsystem.sv | your reference testbenches for the camera and flash IP (register sequences came from here) |
| Makefile, run.sh | build/run targets (system_cam, system_sqz, conv_default, unit, sqz_data ...) |
| filelist_system.f / filelist.f | SoC file list / older compute-only file list |
| CHANGES*.md | change notes per design round (v1..v6) |
| MEM.TXT | default content the flash model loads (overwritten by the testbenches) |

## 10.8 Files no longer used by the SoC

rtl/pixel_converter.sv, rtl/ping_pong_buffer.sv, rtl/ifmap_axi_wrapper.sv (replaced by cam_frontend and act_mem), rtl/accelerator_top_v0.sv (previous compute-only top) and rtl/pe_v2_0.v (older PE). They are kept because the older testbenches reference them.

# 11. Status, limitations and next steps

## Done and verified

SPI control port, register block, interrupt, result pins, back-to-back inferences.

Camera front end with on-chip crop and colour conversion; row-streaming with stripe slots.

Unified activation memory with concurrent camera and compute access.

Weights fetched from QSPI flash in descriptor-sized chunks into a 32 KB weight SRAM.

Compute engine for every SqueezeNet layer type: 3 x 3 stride 1/2, 1 x 1, padding, max-pool (pad 0/1), concat, K up to 1023; classifier head.

## Known limitations

Interconnect: its write path duplicates a beat if a slave pauses WREADY inside a burst. act_mem avoids this (it never pauses writes); the IP should be fixed.

Flash: the quad-enable bit must already be set in the flash (pre-programmed part); the boot flow does not write the status register.

SPI: one word per read, SCK <= clk/8, MISO not tri-stated.

Descriptors: up to 64 jobs held in flip-flops (large); the full network needs a job list in flash.

Camera: the engine must keep up with the camera - an overrun is reported but the frame is then invalid (lower the frame rate if needed). Camera register set-up over SCCB/I2C is left to the MCU.

Older testbenches for the previous top level no longer build; tb_pe expects a 32-bit accumulator.

## Next steps

- Job list fetched from flash; streaming of the Fire expand -> next squeeze stages, pools at fire3/fire5.

- Compiler: fine-tuned ONNX -> jobs + memory plan + flash image.

- Full-network simulation: camera frame -> class pins.

- Fine-tune the 20-30 class model on Colab (ml/finetune_squeezenet.py) and run it.

- FPGA prototype: clocking, block-RAM mapping, board flash offset, OV7670 set-up.

# 12. Glossary

| Term | Meaning |
| --- | --- |
| MAC | multiply-accumulate: acc = acc + a x b |
| Tile | 4 output pixels x 4 output channels computed together by the 4 x 4 array |
| Pass | one run of a layer for 4 output channels |
| Job | one descriptor: a layer, or one stripe of a layer |
| Stripe / halo | a band of image rows processed together / the extra rows it shares with the next band |
| k_len | number of MACs per output value (channels x kernel height x kernel width) |
| Zero point (z_out) | the int8 value that means 'real zero'; -128 for ReLU outputs here |
| XIP | execute-in-place: reading flash as if it were memory |
| Bit-exact | identical to the reference model in every bit |
| LSB | least significant bit - a difference of 1 in the 8-bit result |